MSRS Report Format Documentation

Load Response Regulation Credits

Version 8

Revision History

| DATE | Revision | Description |
| --- | --- | --- |
| 10/19/2007 | 1 | Initial Distribution |
| 8/1/2008 | 2 | Corrected calculation for DSR Reg Lost Opportunity Cost Credit column to MAX(DSR Reg Offer Amount - (DSR PJM-Assigned Reg * Rmcp,0)) |
| 8/22/2012 | 3 | Updated report to add the following columns: DSR Marginal Benefits Factor DSR Performance Score RMCCP ($/MWh) RMPCP ($/MWh) DSR RMCCP Credit ($) DSR RMPCP Credit ($) |
| 10/16/2013 | 4 | Updated report to add the following columns: DSR Mileage Ratio Note: The DSR Marginal Benefits Factor column will be removed from this report effective November 1, 2013. |
| 4/1/2016 | 5 | Updated report to add the following columns: DSR Accuracy Score DSR Delay Score DSR Precision Score |
| 4/1/2018 | 6 | Updated EPT Hour Ending to EPT Interval Ending, GMT Hour Ending to GMT Interval Ending, Updated DSR PJM-Assigned Reg (MWh) to Updated DSR PJM-Assigned Reg MW, Updated DSR Self-Scheduled Reg (MWh) to Updated DSR Self-Scheduled Reg MW; Added supporting calculations section for 5 minute settlements |
| 2/21/2020 | 7 | Removed all references to eSchedule(s) and/or eSchedule(s) reports. |
| 2/13/2024 | 8 | Additional details added to Supported Billing Line Items section regarding counterparty data visibility |

# Report

MSRS Report Name: Load Response Regulation Credits

Report short name for User Interface: Load Response Regulation Credits

Download File Name Abbreviation: LRRegCr

Data Granularity: Sub-hourly

Frequency: Updated monthly

Range Displayed on Report: Start Date through End Date

# Supported Billing Line Items

In order to support reconciliation of the transferred Billing Line Item amount, the “To” Company of a Billing Line Item Transfer may view supporting MSRS report details pertaining to the counterparty for the period spanning the approved Billing Line Item Transfer.

- Regulation and Frequency Response Service Credit (2340)

# Report Content Summary

For each customer account that is a Curtailment Service Provider, this report displays sub-hourly regulation market credits for each registration ID associated with the customer account, and where the RMCCP credit, or Reg Lost Opportunity Cost Credit  associated with the registration ID is greater than 0 for the hour.

# Summary of Changes and Special Logic

- The date range total row will only appear in the online version of the report.  It will not appear in the CSV and XML versions of the report.

- The Report will now display EPT and GMT Hour Ending.

- For 5 Minute Settlements, the report will display EPT and GMT Interval Ending

# Report Columns

The following columns will appear in the body of the report:

| Online and CSV Column Name | XML Column Name | Column Number | Data Type |
| --- | --- | --- | --- |
| Customer ID | CUSTOMER_ID | 4000.01 | INTEGER |
| Customer Code | CUSTOMER_CODE | 4000.02 | VARCHAR2(6) |
| EPT Interval Ending | EPT_INTERVAL_ENDING | 4001.40 | VARCHAR2(40) mm/dd/yyyy HH24:MM format (Displays first interval of the day as hour 0 minute 05 and last interval of the day as hour 24 minute 00) |
| GMT Interval Ending | GMT_INTERVAL_ENDING | 4001.41 | VARCHAR2(40) mm/dd/yyyy HH24:MM format (Displays first interval of the day in relation to EPT interval as hour 04 minute 05 or hour 05 minute 05 (EDT/EST depending) and last interval of the day as hour 04 minute 00 of the next day or hour 05 minute 00 of the next day (EDT/EST depending)) |
| Registration ID | REGISTRATION_ID | 4000.93 | NUMBER(*,0) |
| EDC Account Number | EDC_ACCOUNT_NUMBER | 4000.94 | VARCHAR2(25) |
| End Use Customer | END_USE_CUSTOMER | 4000.95 | VARCHAR2(40) |
| DSR PJM-Assigned Reg MW | DSR_PJM_ASSIGNED_REG_MW | 2340.65 | NUMBER(22,3) |
| DSR Self-Scheduled Reg MW | DSR_SELF_SCHEDULED_REG_MW | 2340.66 | NUMBER(22,3) |
| Online and CSV Column Name | XML Column Name | Column Number | Data Type |
| DSR Mileage Ratio | DSR_MILEAGE_RATIO | 2340.47 | NUMBER |
| DSR Accuracy Score | DSR_ACCURACY_SCORE | 2340.54 | NUMBER |
| DSR Delay Score | DSR_DELAY_SCORE | 2340.55 | NUMBER |
| DSR Precision Score | DSR_PRECISION_SCORE | 2340.56 | NUMBER |
| DSR Performance Score | DSR_PERF_SCORE | 2340.42 | NUMBER |
| RMCCP ($/MWh) | RMCCP | 3001.44 | NUMBER(22,2) |
| RMPCP ($/MWh) | RMPCP | 3001.45 | NUMBER(22,2) |
| DSR RMCCP Credit ($) | DSR_RMCCP_CREDIT | 2340.43 | NUMBER(22,2) |
| DSR RMPCP Credit ($) | DSR_RMPCP_CREDIT | 2340.44 | NUMBER(22,2) |
| DSR Reg Offer Amount ($) | DSR_REG_OFFER_AMOUNT | 2340.28 | NUMBER(22,2) |
| DSR Reg Lost Opportunity Cost Credit ($) | DSR_REG_LOC_CR | 2340.29 | NUMBER(22,2) |
| Version | VERSION | 4000.07 | VARCHAR2(12) |

# CSV Report Example

See Excel file titled “Load Response Regulation Credits CSV Format.csv”

# XML Report Example

See XML file titled “Load Response Regulation Credits XML Format.xml”

# Hyperlinks

The online version of this report does not contain hyperlinks.

# Supporting Calculations

DSR Performance Score (2340.42) = (1/3 * (DSR Accuracy Score (2340.54))) + (1/3 * (DSR Delay Score (2340.55))) + (1/3 * (DSR Precision Score (2340.56)))

If DSR Performance Score (2340.42) <  .25,  then

DSR RMCCP Credit (2340.43) = 0

DSR RMPCP Credit (2340.44) = 0

DSR Reg Lost Opportunity Cost Credit (2340.29) = 0

If DSR Performance Score (2340.42) >=  .25,  then

DSR RMCCP Credit (2340.43) = (DSR PJM-Assigned Reg  (2340.25) + DSR Self-Scheduled Reg (2340.26)) * DSR Marginal Benefits Factor (2340.41) * DSR Performance Score (2340.42) * RMCCP (3001.44)

DSR RMPCP Credit (2340.44) = (DSR PJM-Assigned Reg  (2340.25) + DSR Self-Scheduled Reg (2340.26)) * DSR Marginal Benefits Factor (2340.41) * DSR Performance Score (2340.42) * RMPCP (3001.45)

DSR Reg Lost Opportunity Cost Credit (2340.29) = MAX(DSR Reg Offer Amount (2340.28) - ((DSR PJM-Assigned Reg (2340.25) * DSR Marginal Benefits Factor (2340.41) * DSR Performance Score (2340.42)) * (RMCCP (3001.44) + RMPCP (3001.45))),0)

11/7/2013 Implementation date, effective retroactive to 10/1/2012:

If DSR Performance Score (2340.42) <  .25,  then

DSR RMCCP Credit (2340.43) = 0

DSR RMPCP Credit (2340.44) = 0

DSR Reg Lost Opportunity Cost Credit (2340.29) = 0

If DSR Performance Score (2340.42) >=  .25,  then

DSR RMCCP Credit (2340.43) = (DSR PJM-Assigned Reg  (2340.25) + DSR Self-Scheduled Reg (2340.26)) * DSR Performance Score (2340.42) * RMCCP/ (3001.44)

DSR RMPCP Credit (2340.44) = (DSR PJM-Assigned Reg  (2340.25) + DSR Self-Scheduled Reg (2340.26)) * DSR Mileage Ratio (2340.41) * DSR Performance Score (2340.42) * RMPCP (3001.45)

DSR Reg Lost Opportunity Cost Credit (2340.29) = MAX(DSR Reg Offer Amount (2340.28) - (DSR PJM-Assigned Reg (2340.25) * DSR Performance Score (2340.42) * RMCCP (3001.44)) – (DSR PJM-Assigned Reg (2340.25) * DSR Performance Score (2340.42) * DSR Mileage Ratio * RMPCP (3001.45))),0)

Calculations for 5 Minute Settlements:

If DSR Performance Score (2340.42) <  .25,  then

DSR RMCCP Credit (2340.43) = 0

DSR RMPCP Credit (2340.44) = 0

DSR Reg Lost Opportunity Cost Credit (2340.29) = 0

If DSR Performance Score (2340.42) >=  .25,  then

DSR RMCCP Credit = (DSR PJM-Assigned Reg + DSR Self-Scheduled Reg) * DSR Performance Score * RMCCP/12

2340.43 = (2340.65 + 2340.66) * 2340.42 * 3001.44/12

DSR RMPCP Credit = (DSR PJM-Assigned Reg + DSR Self-Scheduled Reg) * DSR Mileage Ratio * DSR Performance Score * RMPCP/12

2340.44 = (2340.65 + 2340.66) * 2340.41 * 2340.42 * 3001.45/12

DSR Reg Lost Opportunity Cost Credit = MAX(DSR Reg Offer Amount/12 - (DSR PJM-Assigned Reg * DSR Performance Score * RMCCP/12) – (DSR PJM-Assigned Reg * DSR Performance Score * DSR Mileage Ratio * RMPCP/12)),0)

2340.29 = MAX(2340.28/12 – (2340.65 * 2340.42 * 3001.44/12) – (2340.65 * 2340.41 * 2340.42 * 3001.45/12) ,0)