MSRS Report Format Documentation

Regulation Credits

Version 9

Revision History

| DATE | Revision | Description |
| --- | --- | --- |
| 10/19/2007 | 1 | Initial Distribution |
| 8/23/2012 | 2 | Updated report to add the following columns: Marginal Benefits Factor Performance Score RMCCP ($/MWh) RMPCP ($/MWh) RMCCP Credit ($) RMPCP Credit ($) Ramp-In Regulation Lost Opportunity Cost ($) Intra-Hour Regulation Lost Opportunity Cost ($) Ramp-Out Regulation Lost Opportunity Cost ($) Update report to remove the following columns: RT LMP Desired MWh RT Generator LMP ($/MWh) Regulation Lost Opportunity Cost ($) |
| 10/10/2012 | 3 | Updated the Supporting Calculations for Regulation Lost Opportunity Cost Credit for Hydro units. |
| 10/16/2012 | 4 | Updated report to add the following columns: Unit Specific Benefits Factor Updated the Supporting Calculations for Regulation Lost Opportunity Cost Credit for Non-Hydro units. |
| 10/16/2013 | 5 | Updated report to add the following column: Mileage Ratio Note: The Marginal Benefits Factor column will be removed from this report effective November 1, 2013 |
| 4/1/2016 | 6 | Added the following columns to report: Accuracy Score Delay Score Precision Score |
| 4/1/2018 | 7 | Updated EPT Hour Ending to EPT Interval Ending, Updated GMT Hour Ending to GMT Interval Ending, Updated PJM-Assigned Reg (MWh) to PJM-Assigned Reg MW, Updated Self-Scheduled Reg (MWh) to Self-Scheduled Reg MW; Added columns Regulation Opportunity Cost, RT LMP Used, Hydro Average LMP; Added calculations for 5 Minute Settlements to Supporting Calculations Section |
| 2/21/2020 | 8 | Removed all references to eSchedule(s) and/or eSchedule(s) reports. |
| 2/13/2024 | 9 | Additional details added to Supported Billing Line Items section regarding counterparty data visibility |

# Report

MSRS Report Name:  Regulation Credits

Report short name for User Interface: Regulation Credits

Download File Name Abbreviation: RegCr

Data Granularity:  Sub-hourly

Frequency:  Updated daily

Range Displayed on Report:  Start Date through End Date

# Supported Billing Line Items

In order to support reconciliation of the transferred Billing Line Item amount, the “To” Company of a Billing Line Item Transfer may view supporting MSRS report details pertaining to the counterparty for the period spanning the approved Billing Line Item Transfer.

- Regulation and Frequency Response Service Credit (2340)

# Report Content Summary

This report displays the customer account’s hourly Regulation Market Capability Clearing Price Credit, Regulation Market Performance Clearing Price Credits and Regulation Lost Opportunity Cost Credit where the customer’s RMCCP Credit, RMPCP Credit or Reg Lost Opportunity Cost Credit is not equal to 0.  The credits in this report do not reflect the customer account’s share of jointly owned units. All owners will see the full credit assigned to the unit.

# Summary of Changes and Special Logic

- The date range total row will only appear in the online version of the report.  It will not appear in the CSV and XML versions of the report.

- The Report will now display EPT and GMT Hour Ending.

- For 5 Minute Settlements the report will display EPT and GMT Interval Ending

- If the unit is a hydro unit, the Hydro Spill Indicator will indicate whether the unit was spilling or not for the given hour; If the unit is a non-hydro unit the Hydro Spill Indicator will display as null.

# Report Columns

The following columns will appear in the body of the report:

| Online and CSV Column Name | XML Column Name | Column Number | Data Type |
| --- | --- | --- | --- |
| Customer ID | CUSTOMER_ID | 4000.01 | INTEGER |
| Customer Code | CUSTOMER_CODE | 4000.02 | VARCHAR2(6) |
| EPT Interval Ending | EPT_INTERVAL_ENDING | 4001.40 | VARCHAR2(40) mm/dd/yyyy HH24:MM format (Displays first interval of the day as hour 0 minute 05 and last interval of the day as hour 24 minute 00) |
| GMT Interval Ending | GMT_INTERVAL_ENDING | 4001.41 | VARCHAR2(40) mm/dd/yyyy HH24:MM format (Displays first interval of the day in relation to EPT interval as hour 04 minute 05 or hour 05 minute 05 (EDT/EST depending) and last interval of the day as hour 04 minute 00 of the next day or hour 05 minute 00 of the next day (EDT/EST depending)) |
| Unit ID | UNIT_ID | 4000.63 | NUMBER(8,0) |
| Unit Name | UNIT_NAME | 4000.64 | VARCHAR2(60) |
| Online and CSV Column Name | XML Column Name | Column Number | Data Type |
| Unit Ownership Share | UNIT_OWNERSHIP_SHARE | 3000.8 | NUMBER |
| PJM-Assigned Reg MW | PJM_ASSIGNED_REG_MW | 2340.63 | NUMBER(22,3) |
| Self-Scheduled Reg MW | SELF_SCHEDULED_REG_MW | 2340.64 | NUMBER(22,3) |
| Mileage Ratio | MILEAGE_RATIO | 2340.46 | NUMBER |
| Unit Specific Benefits Factor | UNIT_BENEFIT_FACTOR | 2340.45 | NUMBER |
| Accuracy Score | ACCURACY_SCORE | 2340.51 | NUMBER |
| Delay Score | DELAY_SCORE | 2340.52 | NUMBER |
| Precision Score | PRECISION_SCORE | 2340.53 | NUMBER |
| Performance Score | PERF_SCORE | 2340.35 | NUMBER |
| RMCCP ($/MWh) | RMCCP | 3001.44 | NUMBER(22,2) |
| RMPCP ($/MWh) | RMPCP | 3001.45 | NUMBER(22,2) |
| RMCCP Credit ($) | RMCCP_CREDIT | 2340.36 | NUMBER(22,2) |
| RMPCP Credit ($) | RMPCP_CREDIT | 2340.37 | NUMBER(22,2) |
| Bias Factor | BIAS_FACTOR | 2340.20 | NUMBER |
| Online and CSV Column Name | XML Column Name | Column Number | Data Type |
| Hydro Spill Indicator | HYDRO_SPILL_INDICATOR | 4000.67 | VARCHAR2(1) See possible values below |
| Reg Offer Price ($/MWh) | REG_OFFER_PRICE | 2340.21 | NUMBER(14,2) |
| RT LMP Used ($/MWh) | RT_LMP_USED | 2340.61 | NUMBER(22,2) |
| Hydro Average LMP ($/MWh) | HYDRO_AVG_LMP | 2340.62 | NUMBER(22,2) |
| Reg Offer Amount ($) | REG_OFFER_AMOUNT | 2340.22 | NUMBER |
| Ramp-In Regulation Lost Opportunity Cost ($) | RAMP_IN_REG_LOC | 2340.38 | NUMBER(22,2) |
| Intra-Hour Regulation Lost Opportunity Cost ($) | INTRA_HOUR_REG_LOC | 2340.39 | NUMBER(22,2) |
| Ramp-Out Regulation Lost Opportunity Cost ($) | RAMP_OUT_REG_LOC | 2340.40 | NUMBER(22,2) |
| Regulation Opportunity Cost ($) | REG_OPP_COST | 2340.60 | NUMBER(22,2) |
| Regulation Lost Opportunity Cost Credit ($) | REG_LOC_CREDIT | 2340.24 | NUMBER |
| Version | VERSION | 4000.07 | VARCHAR2(12) |

Possible Hydro Spill Indicator Values: Y or N

# CSV Report Example

See Excel file titled “Regulation Credits CSV Format.csv”

# XML Report Example

See XML file titled “Regulation Credits XML Format.xml”

# Hyperlinks

The online version of this report does not contain hyperlinks.

# Supporting Calculations

Performance Score (2340.35) = (1/3 * Accuracy Score (2340.51)) + (1/3 * Delay Score (2340.52)) + (1/3 * Precision Score (2340.53))

If Performance Score (2340.35) <  .25,  then

RMCCP Credit (2340.36) = 0

RMPCP Credit (2340.37) = 0

Reg Offer Amount (2340.22) = 0

Regulation Lost Opportunity Cost Credit (2340.24) = 0

If Performance Score (2340.35) >=  .25,  then

RMCCP Credit (2340.36) = (PJM-Assigned Reg (2340.17) + Self-Scheduled Reg (2304.18)) * Marginal Benefits Factor (2340.34) * Performance Score (2340.35) * RMCCP (3001.44)

RMPCP Credit (2340.37) = (PJM-Assigned Reg (2340.17) + Self-Scheduled Reg (2304.18)) * Marginal Benefits Factor (2340.34) * Performance Score (2340.35) * RMPCP (3001.45)

Reg Offer Amount (2340.22) = PJM-Assigned Reg (2340.17) * Reg Offer Price (2340.21)

Non-Hydro Units:

Regulation Lost Opportunity Cost Credit (2340.24) = MAX((Ramp-In Regulation Lost Opportunity Cost (2340.38) + (Intra-Hour Regulation

Lost Opportunity Cost (2340.39) * Unit Specific Benefits Factor (2340.45) * Performance Score (2340.35))  + Ramp-Out Regulation Lost

Opportunity Cost (2340.40) + Reg Offer Amount (2340.22)) – ((PJM-Assigned Reg (2340.17) * Marginal Benefits Factor (2340.34) *

Performance Score (2340.35))  * (RMCCP (3001.44) + RMPCP (3001.45))),0)

Hydro Units:

Regulation Lost Opportunity Cost Credit (2340.24) = MAX((Ramp-In Regulation Lost Opportunity Cost (2340.38) + Intra-Hour Regulation

Lost Opportunity Cost (2340.39) + Ramp-Out Regulation Lost Opportunity Cost (2340.40) + Reg Offer Amount (2340.22)) – ((PJM-Assigned

Reg (2340.17) * Marginal Benefits Factor (2340.34) * Performance Score (2340.35))  * (RMCCP (3001.44) + RMPCP (3001.45))),0)

11/7/2013 Implementation date, effective retroactive to 10/1/2012:

If Performance Score (2340.35) <  .25,  then

RMCCP Credit (2340.36) = 0

RMPCP Credit (2340.37) = 0

Reg Offer Amount (2340.22) = 0

Regulation Lost Opportunity Cost Credit (2340.24) = 0

If Performance Score (2340.35) >=  .25,  then

RMCCP Credit (2340.36) = (PJM-Assigned Reg (2340.17) + Self-Scheduled Reg (2304.18)) * Performance Score (2340.35) * RMCCP (3001.44)

RMPCP Credit (2340.37) = (PJM-Assigned Reg (2340.17) + Self-Scheduled Reg (2304.18)) * Mileage Ratio (2340.46) * Performance Score (2340.35) * RMPCP (3001.45)

Reg Offer Amount (2340.22) = PJM-Assigned Reg (2340.17) * Reg Offer Price (2340.21)

Non-Hydro Units:

Regulation Lost Opportunity Cost Credit (2340.24) = MAX((Ramp-In Regulation Lost Opportunity Cost (2340.38) + (Intra-Hour Regulation

Lost Opportunity Cost (2340.39) * Unit Specific Benefits Factor (2340.45) * Performance Score (2340.35)) + Ramp-Out Regulation Lost

Opportunity Cost (2340.40) + Reg Offer Amount (2340.22)) – (PJM-Assigned Reg (2340.17) *

Performance Score (2340.35) * RMCCP (3001.44)) – (PJM-Assigned Reg (2340.17) * Performance Score (2340.35) *Mileage Ratio (2340.46) * RMPCP (3001.45))),0)

Hydro Units:

Regulation Lost Opportunity Cost Credit (2340.24) = MAX((Ramp-In Regulation Lost Opportunity Cost (2340.38) + Intra-Hour Regulation

Lost Opportunity Cost (2340.39) + Ramp-Out Regulation Lost Opportunity Cost (2340.40) + Reg Offer Amount (2340.22)) – (PJM-Assigned Reg (2340.17) * Performance Score (2340.35) * RMCCP (3001.44)) – (PJM-Assigned Reg (2340.17) * Performance Score (2340.35) *Mileage Ratio (2340.46) * RMPCP (3001.45))),0)

Calculations for 5 Minute Settlements:

If Performance Score (2340.35) <  .25,  then

RMCCP Credit (2340.36) = 0

RMPCP Credit (2340.37) = 0

Reg Offer Amount (2340.22) = 0

Regulation Lost Opportunity Cost Credit (2340.24) = 0

If Performance Score (2340.35) >=  .25,  then

RMCCP Credit = (PJM-Assigned Reg + Self-Scheduled Reg) * Performance Score * RMCCP/12

2340.36 = (2340.63 + 2340.64) * 2340.35 * 3001.44/12

RMPCP Credit = (PJM-Assigned Reg + Self-Scheduled Reg) * Mileage Ratio * Performance Score * RMPCP/12

2340.37 = (2340.63 + 2340.64) * 2340.46 * 2340.35 * 3001.45/12

Non-Hydro Units:

Reg Offer Amount = PJM-Assigned Reg * Reg Offer Price

2340.22 = 2340.63 * 2340.21

Regulation Opportunity Cost = (Intra-Hour Regulation Lost Opportunity Cost * Unit Specific Benefits Factor * Performance Score) + Ramp-in Regulation Lost Opportunity Cost + Ramp-Out Regulation Lost Opportunity Cost

*Note – the calculation for Regulation Opportunity Cost assumes the regulating unit was cleared for the full five minute interval the opportunity cost is applied to.  In the event the unit was not cleared for the full five minute interval, the calculation result will be pro-rated based on the percentage of the five minute interval the unit was cleared for.

2340.60 = (2340.39 * 2340.45 * 2340.35) + 2340.38 + 2340.40

Regulation Lost Opportunity Cost Credit = MAX ((Reg Offer Amount + Regulation Opportunity Cost)/12 – (PJM-Assigned Reg * Performance Score * RMCCP/12) – (PJM-Assigned Reg * Performance Score * Mileage Ratio * RMPCP/12) , 0)

2340.24 = MAX((2340.22 + 2340.60)/12 – (2340.63 * 2340.35*3001.44/12) – (2340.63 * 2340.35 * 2340.46 * 3001.45/12) , 0)

Hydro Units:

Reg Offer Amount = PJM-Assigned Reg * Reg Offer Price

2340.22 = 2340.63 * 2340.21

If Spill Indicator = N Then

If DA MW > 0 for the hour corresponding to the 5-Minute Interval, Then

Regulation Opportunity Cost = MAX ([(1-Bias Factor) * PJM-Assigned Reg * Unit Specific Benefits Factor * Performance Score] * (RT LMP Used – Hydro Average LMP), 0)

2340.60 = MAX([(1 – 2340.20) * 2340.63 * 2340.45 * 2340.35] * (2340.61 – 2340.62) ,0)

Else

Regulation Opportunity Cost = MAX ([(1-Bias Factor) * PJM-Assigned Reg * Unit Specific Benefits Factor * Performance Score] * (Hydro Average LMP – RT LMP Used), 0)

2340.60 = MAX([(1 – 2340.20) * 2340.63 * 2340.45 * 2340.35] * (2340.62 – 2340.61) ,0)

Else

Regulation Opportunity Cost = [(1-Bias Factor) * PJM-Assigned Reg * Unit Specific Benefits Factor * Performance Score] * RT LMP Used

2340.60 = [(1 – 2340.20) * 2340.63 * 2340.45 * 2340.35] * 2340.61

Regulation Lost Opportunity Cost Credit = Max ((Reg Offer Amount + Regulation Opportunity Cost)/12 - (PJM-Assigned Reg * Performance Score * RMCCP/12) – (PJM-Assigned Reg * Performance Score * Mileage Ratio * RMPCP/12) , 0)

2340.24 = MAX((2340.22 + 2340.60)/12 – (2340.63 * 2340.35*3001.44/12) – (2340.63 * 2340.35 * 2340.46 * 3001.45/12) , 0)