MSRS Report Format Documentation

Load Response Tier 2 Credits

Version 3

Revision History

| Date | Revision | Description |
| --- | --- | --- |
| 10/19/2007 | 1 | Initial Distribution |
| 4/1/2018 | 2 | Updated GMT/EPT hour ending to GMT/EPT interval ending, calculations |
| 2/21/2020 | 3 | Removed all references to eSchedule(s) and/or eSchedule(s) reports |

# Report

MSRS Report Name: Load Response Tier 2 Credits

Report short name for User Interface: Load Response Tier 2 Credits

Download File Name Abbreviation: LRTier2Cr

Data Granularity: Sub-hourly

Frequency: Updated daily

Range Displayed on Report: Start Date through End Date

# Supported Billing Line Items

- Synchronized Reserve Credit (2360)

# Report Content Summary

For each customer account that is a Curtailment Service Provider, this report displays sub-hourly synchronized reserve tier 2 credits (Synchronized Reserve Market Clearing Price (SRMCP) Credit, Synchronized Reserve Lost Opportunity Cost Credits Cleared and Synchronized Reserve Lost Opportunity Cost Credits Added) for each registration ID associated with the customer account, and where the DSR SRMCP Credit or the DSR Synch Reserve Lost Opportunity Cost Credit Cleared or the DSR Synch Reserve Lost Opportunity Cost Credit Added associated with the registration ID are greater than 0 for the interval.

# Summary of Changes and Special Logic

- The date range total row will only appear in the online version of the report.  It will not appear in the CSV and XML versions of the report.

# Report Columns

The following columns will appear in the body of the report:

| Online and CSV Column Name | XML Column Name | Column Number | Data Type |
| --- | --- | --- | --- |
| Customer ID | CUSTOMER_ID | 4000.01 | INTEGER |
| Customer Code | CUSTOMER_CODE | 4000.02 | VARCHAR2(6) |
| EPT Interval Ending | EPT_INTERVAL_ENDING | 4001.40 | VARCHAR2(40) mm/dd/yyyy HH24:MM format (Displays first interval of the day as hour 0 minute 05 and last interval of the day as hour 24 minute 00) |
| GMT IntervalEnding | GMT_INTERVAL_ENDING | 4001.41 | VARCHAR2(40) mm/dd/yyyy HH24:MM format Displays first interval of the day in relation to EPT interval as hour 04 minute 05 or hour 05 minute 05 (EDT/EST depending) and last interval of the day as hour 04 minute 00 of the next day or hour 05 minute 00 of the next day (EDT/EST depending) |
| Registration ID | REGISTRATION_ID | 4000.93 | NUMBER(*,0) |
| EDC Account Number | EDC_ACCOUNT_NUMBER | 4000.94 | VARCHAR2(25) |
| End Use Customer | END_USE_CUSTOMER | 4000.95 | VARCHAR2(40) |
| SRMCP ($/MWh) | SRMCP | 3000.61 | NUMBER(14,2) |
| DSR Tier 2 PJM-Scheduled MW | DSR_TIER2_PJM_SCHEDULED_MW | 2360.40 | NUMBER(22,3) |
| DSR Tier 2 PJM-Added MW | DSR_TIER2_PJM_ADDED_MW | 2360.41 | NUMBER(22,3) |
| DSR Tier 2 Self-Scheduled MW | DSR_TIER2_SELF_SCHEDULED_MW | 2360.42 | NUMBER(22,3) |
| DSR Tier 2 Shortfall (MW) | DSR_TIER2_SHORTFALL | 2360.43 | NUMBER(22,2) |
| DSR SRMCP Credit ($) | DSR_SRMCP_CR | 2360.44 | NUMBER(22,2) |
| DSR Synch Reserve Offer Amount ($) | DSR_SYNCH_RES_OFFER_AMOUNT | 2360.45 | NUMBER(14,2) |
| DSR Synch Reserve Lost Opportunity Cost Credit Cleared ($) | DSR_SYNCH_RES_LOC_CR_CLEARED | 2360.46 | NUMBER(22,2) |
| DSR Synch Reserve Lost Opportunity Cost Credit Added ($) | DSR_SYNCH_RES_LOC_CR_ADDED | 2360.47 | NUMBER(22,2) |
| Version | VERSION | 4000.07 | VARCHAR2(12) |

# CSV Report Example

See Excel file titled “Load Response Tier 2 Credits CSV Format.csv”

# XML Report Example

See XML file titled “Load Response Tier 2 Credits XML Format.xml”

# Hyperlinks

The online version of this report does not contain hyperlinks.

# Supporting Calculations

DSR SRMCP Credit (2360.44) = SRMCP (3000.61) * (DSR Tier 2 PJM-Scheduled MWh (2360.40) + DSR Tier 2 PJM-Added MWh (2360.41) + DSR Tier 2 Self-Scheduled MWh (2360.42) - DSR Tier 2 Shortfall (2360.43))

DSR Synch Reserve Lost Opportunity Cost Credit Cleared (2360.46) = MAX (DSR Synch Reserve Offer Amount (2360.45) – DSR SRMCP Credit (2360.44), 0)

Calculations for 5 Minute Settlements:

DSR SRMCP Credit = SRMCP * (DSR Tier 2 PJM-Scheduled MWh + DSR Tier 2 PJM-Added MWh + DSR Tier 2 Self-Scheduled MWh - DSR Tier 2 Shortfall)

(2360.44) = [(3000.61) * ((2360.40) + (2360.41) + (2360.42) - (2360.43))] / 12

DSR Synch Reserve Lost Opportunity Cost Credit Cleared = MAX (DSR Synch Reserve Offer Amount – DSR SRMCP Credit, 0)

(2360.46) = MAX((2360.45) - (2360.44)),0)